APPLE: Find My Certification

- Enrolment

Requirements:

- Registered company

- Join MFi program https://mfi.apple.com/

- DUNS number, Tax identification num, business verification

- Website and mail

- Problem statement of product

- Non disclosure agreement

- Hardware requirements: rough prototype, ble advertisement spec, power

Outcomes

- Access to private technical specs like FM BLE Format, encryption keys

- Authorization to design FM compatible products

- Will be able to use Apples encryption tech

- Product requirements

Device must follow:

- Use Apple-approved BLE encryption (ATECC608B)

Need to pass Apple's encryption requirements. ESP32-S3 alone might not be sufficient. If skipped, Apple can reject at security review phase.

- Power saving guidelines

- Secure element chip

- Pass the audits

- Testing and certification

- prototype and lab testing

GOOGLE Certification

https://developers.google.com/nearby/fast-pair/specifications/extensions/fmdn

Google Fast Pair, Audio Switch & Find My Device Proposal Form

User can track the lost devices using any android phone.

Assumptions: WiFi + Bluetooth + GPS

Certification will give us Google’s security, privacy, and technical requirements and our device will be accepted in the Find My Device Network.

Prerequisites:

- Device supports Fast pair (ISSUE - If our device is not natively supporting Fast Pair, you will need to clarify the custom implementation and whether you have additional libraries or SDKs for it.

implement the Fast Pair protocol using BLE, cryptography, and GATT characteristics: WILL BE DONE BY PROGRAMMING EPS32S3

It will require custom development and integration of cryptographic methods for HMAC-SHA256 and AES-128.

You will need to test your implementation and ensure compliance with Google’s Fast Pair requirements before submitting for certification

- Impleement FMDN’s (Find My Device Network) new GATT characteristic and authentication mechanism.

- Submit the google proposal form

Technical Implementation

- Add GATT characteristic

Add characteristic UUID and give Read, Write, Notify permissions

- Implement all the cryptography: HMAC-SHA256, AES-128, and secure key storage on flash/NVS.

- Handle all failure conditions (authentication failure, GATT errors).

- Show test logs proving it works.

- Error Handling

- BLE Advertising update

- Support authentication flow

- When a Seeker (user’s phone) reads the characteristic, your device must send:

A random nonce (8 bytes) + protocol version (1 byte = 0x01)

- When a Seeker writes back, it uses HMAC-SHA256 with keys like:

- Account Key

- Owner Account Key

- Ephemeral Identity Key (EIK)

- Recovery Key / Ring Key / Unwanted Tracking Key (all derived from EIK)

- Implement all required operations:

- Read beacon parameters

- Read provisioning state

- Set/Clr EIK

- Ringing

- Key recovery

- Enable tracking protection

- Sends notis

- Certi and Validation

- Google team will review the device proposal

- Submit test report or device

- Pass the tests

- And once certified our device will be able to work with Androids FDMN

- Users will be able to see tracker in Android Find My Device App

| Section | What You Must Provide |
| --- | --- |
| 1. Contact Information | Your company name, official email ID (best if it’s a company domain, e.g., xyz@yourcompany.com), contact person name, phone number. |
| 2. Company Information | Company legal name, headquarters address, website link, your D-U-N-S number (optional, but good if you have). |
| 3. Product Details | Device name (example: "SmartTag S3"), Product category (example: "Tracker"), Hardware platform (example: "ESP32-S3"), Firmware version (example: "v1.0.0"), model number (optional). |
| 4. Fast Pair Information | - Already certified Fast Pair Model ID? (If yes, give Model ID) - If no, apply for Model ID first separately (Fast Pair requires a unique Model ID). |
| 5. FMDN Extension Implementation Details | - Confirm that you support FMDN GATT Characteristics. - Confirm crypto (HMAC-SHA256) support. - Confirm secure key management (Account Key, EIK, Recovery Key, etc.). - Confirm BLE Advertising format compliance. - Confirm authentication flow is implemented. |
| 6. Certification/Validation | - Confirm you will submit a device sample for testing or submit full test logs/screenshots. - Confirm you agree to Google's security/privacy policies. |
| 7. Additional Information (Optional) | Anything extra you want to tell — like special features your device has, or any deviations you request. |